DC-D4516WRX 2.8mm / 4.0mm

Architectural and Engineering Specifications

Version 1.0

(Apr. 26, 2023)

PART 1: PLEASE REFER TO ATTACHED DOCUMENTS - OVERVIEW & FORMAT SAMPLES

PART 2: PRODUCTS

Division 28 – Electric Safety and Security

Level 1 - 28.20.00 – Video Surveillance

Level 2 - 28.21.00 – Surveillance Cameras

Level 3 - 28.21.13 – IP Cameras

Manufacturer

- IDIS Co., Ltd.
IDIS Tower, 344 Pangyo-ro, Bundang-gu
Seongnam-si, Gyeonggi-do, 13493, Korea
Tel: 	+82 31 723 5400
Fax: 	+82 31 723 5100

General

Product Description

DC-D4516WRX is an outdoor Dome type IP Camera designed and manufactured by IDIS. This camera provides 5MP (2592 x 1944) resolution at 30ips (images per second) with H.265, H.264, and M-JPEG compression. This camera is equipped with IR LEDs, True Day/Night, PoE (IEEE 802.3af Class 2), Alarm I/O, Audio I/O and microSD/SDHC/SDXC card backup.

General Specification

- The IP camera shall be equipped with 2 Megapixel 1/2.8” CMOS Sensor.

- The IP camera shall be equipped with 2.8mm/4.0mm fixed-focal lens, F2.0 (2.8mm) / F2.15 (4.0mm).

- The IP camera shall be a true day/night camera with a mechanical filter for low light performance.
The filter can be switched remotely, or automatically via a light level sensor or contact input (ICR).

- The IP camera shall have wide dynamic range compensation (True WDR) for improved video quality in high-contrast situations (120 dB).

- The IP camera shall be equipped with 4ea Infrared LEDs ranging up to 30m/98.4ft

- The IP camera shall support fixed Iris.

- The IP camera shall be equipped with 10/100 Base-T, auto-sensing, half/full duplex, RJ-45 Ethernet connection.

- The IP camera shall support industry standard Power over Ethernet (PoE) IEEE 802.3af, Class 3.

- The IP camera shall have video out feature (NTSC/PAL).

- Using IDIS NLTSrec(Non-Linear Time Shifting recording) technology, the IP camera can store the recording data to the internal recording memory buffer (60MB) in camera if there is a delay in data transmission due to the instantaneous load of the recorder or network, and then transmits the stored data to IDIS recorder safely.

- The IP camera shall deliver maximum video resolution of 2592x1944 at rates up to 30ips (images per second).

- The IP camera shall provide network connection using H.265, H.264 and MJPEG compression.

- The IP camera shall support quadruple streams in DirectIP 2.0 protocol mode.

- The IP camera shall conform to the ONVIF Profile S.

- The IP camera shall be equipped with embedded web server (IDIS Web) which works independently using a Web Browser with ActiveX plug-in.

- The IP camera shall support SSL Encryption, Multi-user Authority, IEEE 802.1x IP Filtering, HTTPS for greater security.

- The IP camera shall have network bandwidth limitation and MAT features for more efficient use of network bandwidth.

- The IP camera shall have Easy network access via UPnP (Universal Plug and Play) function and embedded mDNS (Multicast DNS) protocol.

- The IP camera shall have Intelligent Video Analysis (VA): Video Motion Detection, Active Tampering Alarm, Trip-Zone.

Protocol Specification: DirectIP 2.0

- The IP camera shall have DirectIP 2.0 mode.

- DirectIP 2.0 protocol shall provide easy connection to DirectIP NVR for automatic discovery and video streaming configuration.

- DirectIP 2.0 shall provide the compatibility with IDIS Solution Suite VMS or ONVIF for third-party software solutions.

- DirectIP 2.0 shall support camera can be linked to IDIS software solution such ad IDIS Center and IDIS Solution Suite, or 3rd party solution while it is being connected to a DirectIP NVR.

- DirectIP 2.0 camera shall be compatible with DirectIP 1.0 NVR as well as DirectIP 2.0 NVR.

- DirectIP 2.0 camera shall be unavailable for No-password login when connecting to DirectIP 2.0 NVR and IDIS Software Solutions.

- DirectIP 2.0 protocol shall provide Quadruple streams.

- DirectIP 2.0 protocol shall support H.264 and H.265 and MJPEG compression.

Technical Specification

Video Specification

- Image Sensor: 1/2.8” CMOS

- Maximum Resolution: 2592x1944

- Scanning Mode: Progressive Scan

- Lens Type: Fixed-focal (f=2.8mm/4.0mm, F2.0/2.15)

- Iris Control: Fixed Iris

- Angular Field of View (H: Horizontal, V: Vertical, D:Diagonal):

  - f=2.8mm : 104.8º (H) 76.9º(V) 130º(D)

  - f=4.0mm : 79.2º (H) 56.6º(V) 103º(D)

- Pan/Tilt Range: Pan: -170° ~ 170°, Tilt: -5° ~ 67.5°, Rotate: -109° ~ 109°

- Minimum Illumination:

  - f=2.8mm : COLOR: 0.15 lux @ F2.0 / B/W : 0 lux  (IR LED ON)

  - f=4.0mm : COLOR: 0.15 lux @ F2.15 / B/W : 0 lux  (IR LED ON)

- S/N Ratio: more than 45 dB

- Maximum Frame Rate:

  - 4:3 mode : 30ips : 2592 x 1944(WDR)

  - 16:9 mode : 30ips : 2592 x 1456(WDR)

- Video Resolution:

  - 4:3 : 2592x1944, 1920x1440, 1280x960x 640x480

  - 16:9 : 2592x1456, 1920x1080, 1280x720, 640x360

- Video Compression : H.265, H.264, MJPEG

- Video Compression Level: 4 levels: Basic, Standard, High, Very High

- Intelligent Codec is supported.

- Multi-Video Streaming: Quadruple streams

- Dynamic Range: 120dB

- True Day & Night: Yes (ICR)

  - IR Distance (The number of LEDs, IR wavelength): 30m /98.4ft (4ea)

- Intelligent Video Analytic: Video Motion Detection, Active Tampering Alarm, Trip-Zone.

Network Specification

- Port: RJ-45 10/100 Base-T 1 port

- Network Protocols: DirectIP 2.0 Protocol, IPv4, IPv6, RTP/RTSP/TCP, RTP/RTSP/HTTP/TCP, RTP/UDP RTSP/TCP, HTTP, HTTPS, FTP, SNTP, SMTP, FEN, mDNS, uPNP, SNMPv2

- Streaming Mode: Quadruple Streaming

Security Specification

- SSL Encryption, Multi-User Authority, IEEE 802.1x, IP Filtering, HTTPS

Audio Specification

- Audio Compression Algorithm: ADPCM (16kHz), G.726, G.711 u/a-Law

- Audio Input / Output: Line-in 1ea + Internal Mic 1ea

- Two-way Audio Communication: Yes

- Pre-recorded Voice Alert: Yes

Alarm and Event Specification

- Trigger Events: Motion Detection, Alarm in, Tampering, Trip Zone

- Event Notification: Remote S/W, Email (with Image)

Environmental Specification

- Operating Temperature: -30°C ~ +55°C (-22°F ~ +131°F)

- Starting up at above 0°C (32°F)

- Operating Humidity: 0% to 90% non-condensing

- Vandal Proof: IK10

- International Protection Marking: IP67

Electrical Specification

- Power Source: PoE (IEEE 802.3af class 3)

- Power Consumption: PoE, IEEE 802.3af Class 3, 7.6W

- Regulatory Approvals: FCC, CE, KC

Mechanical Specification

- Dimensions (Ø x H): Ø120.6 x 92.9mm

- Unit Weight: 0.48kg(1.06lb)

- Version History

| Version | Writer | Revision Date | Remarks |
| --- | --- | --- | --- |
| 1.0 | TS Team | Apr. 26, 2023 | Initial Release |